March 10, 2006

Jacquelyn Brown

Department of Public Works

Room G-4

165 Capital Avenue

Hartford, CT  06106

## RE: 25 Sigourney Street, Hartford, CT

## Sheetrock Removal & Wall Investigation

Project # 44013-0200-00000

Dear Ms. Brown:

TRC Environmental (TRC) was contacted to assist in the oversight and monitoring of sheetrock removal at 25 Sigourney Street, Hartford, CT. This project was initiated due to a previous consultants’ report. The scope of this project includes: the identification and demarcation of suspect water impacted building materials, subsequent material removal, inspection and monitoring of the removal process. TRC sent Martin Lewis, C.I.H., Jonathan Fitzsimonds (Project Scientist) & Justin Corso (Project Scientist) to the site on three occasions between February 10th &12th, 2006.

On Friday February 10, 2006 TRC began the project by meeting with Connecticut Department of Public Works (CTDPW) representative Donna Baisley to discuss the scope of work required and work schedule.

TRC’s responsibilities include; physical identification of each area of concern within the building, Clearly demarcate the scope of removal at each location, record material moisture content, monitor and record airborne particulate levels prior to and throughout the removal process. TRC utilized state of the art data collection equipment including: a GE Infrastructure Sensing, Surveymaster™ Protimeter Dual-Function Moisture Meter, TSI DustTrak Aerosol Monitor (model # 8520) and an MIE PDR1000 Airborne Particulates Monitor.

Following the meeting, each location was identified and marked based upon thermal imaging data provided by the CTDPW. Please refer to Infrared Inspection Report, (not included) produced by Monroe Infrared Technology for 25 Sigourney Street Hartford, Connecticut, dated October 10, 2005 for a complete index of infrared photographs and location diagrams.  At the time of initial identification and demarcation, TRC conducted airborne particulate monitoring and substrate moisture content sampling.

Sample results indicate moisture content averaged below 1.0 for surface moisture and 11.0 for subsurface moisture on the Wood Moisture Equivalent (WME) Scale, with highest readings of 9.4 and 16.0 respectively. Airborne particulates samples taken prior to any removal activities were below OSHA standards, averaging approximately 0.007mg/m³ (TSI DustTrak)/0.009mg/m³ (MIE PDR) with  highest readings of 0.015mg/m³ and 0.016mg/m³ respectively. Airborne particulates samples taken during removal activities were also below OSHA standards, averaging approximately 0.03mg/m³ (MIE PDR) with a highest reading of 0.138mg/m³. See Appendix A-SAMPLE RESULTS TABLES for a complete list of sample results.

Removal of identified wallboard/sheetrock began on Saturday February 11, 2006 and continued through Sunday February 12, 2006. Haz-Pros Environmental contractors (Haz-Pros) performed the work of removing sheetrock from the walls in forty (40) of the forty nine (49) identified locations over twelve (12) floors. Those areas not incuded were outside the scope of wallboard/sheetrock removal. At each location Haz-Pros utilized a HEPA filtered micro-trap prior to any disturbance of the material. Haz-Pros then covered any horizontal surfaces in the surrounding area with six (6) mil polyethylene sheeting and cut the material away with razor knives and keyhole saws. Sheetrock debris was bagged tape sealed and removed from the site. Clean-up was done through the use of HEPA filtered vacuums and wet wiping surfaces. Following an acceptable visual inspection each area was then sealed with a temporary barrier of six (6) mil polyethylene sheeting and high strength duct tape. The duct tape was then stapled to the remaining sheetrock to ensure adhesion. No spray glue was used on this site.

Observations:

- Wall moisture content was found to be below acceptable levels based upon the WME scale.

- Particulate concentrations prior to and during the removal were all within OSHA standards for total airborne particulates.

- No visible mold growth was observed on wallboard surfaces.

Thank you again for the opportunity to be of service.  If you have any questions, please call me at (860) 298-6350.

Sincerely,

TRC ENVIRONMENTAL CORPORATION

Jonathan P. Fitzsimonds

Project Scientist

TRC Corporation

Appendix A

SAMPLE RESULTS TABLES

| Wall Moisture and Airborne Particulate Sample Results |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Location ID# | Location/ Area | % Water WME (surface/ subsurface) | Particulate Concentrations Pre-Removal (mg/m³) |  | Particulate Concentrations During Removal (mg/m³) |
|  |  |  | TSI DustTrak | MIE PDR | MIE PDR |
| 1 | 19th Floor Room 1913 | 0/13 | 0.006 | 0.013 | 0.013 |
| 2 | 19th Floor Room 1920 | 0/16 | 0.003 | 0.013 | 0.087 |
| 3 | Floor Area | Not In Scope |  |  |  |
| 4 | 19th Floor Room 1932 | 0/11 | 0.003 | 0.008 | 0.032 |
| 5 | 18th Floor Room 1805 | 0/11 | 0.010 | 0.011 | 0.016 |
| 6 | Ceiling Area | Not In Scope |  |  |  |
| 7 | 18th Floor Room 1806 | 8.5/15 | 0.009 | 0.010 | 0.030 |
| 8 | 18th Floor Joe Thomas’ Office | 0/9 | 0.004 | 0.008 | 0.076 |
| 9 | 18th Floor Joe Thomas’ Office | 9.4/11 | 0.004 | 0.008 | 0.076 |
| 10 | 18th Floor Room 1809 | 0/11 | 0.004 | 0.008 | 0.029 |
| 11 | 18th Floor Room 1810 | 0/11 | 0.003 | 0.006 | 0.015 |
| 12 | Ceiling Area | Not In Scope |  |  |  |
| 13 | 18th Floor Outside Room 1810 | 8.5/11 | 0.008 | 0.016 | 0.027 |
| Wall Moisture and Airborne Particulate Sample Results |  |  |  |  |  |
| Location ID# | Location/ Area | % Water WME (surface/ subsurface) | Particulate Concentrations Pre-Removal (mg/m³) |  | Particulate Concentrations During Removal (mg/m³) |
|  |  |  | TSI DustTrak | MIE PDR | MIE PDR |
| 14 | 18th Floor (T Grove) | 0/11 | 0.006 | 0.007 | 0.075 |
| 15 | 18th Floor (N Carola) | 0/11 | 0.015 | 0.007 | 0.027 |
| 16 | 18th Floor (J Vuocolo) | 0/13 | 0.013 | 0.009 | 0.011 |
| 17 | Ceiling Area | Not In Scope |  |  |  |
| 18 | 18th Floor Room 1826 | 0/11 | 0.008 | 0.007 | 0.021 |
| 19 | 17th Floor Room 1706 | 0/11 | 0.008 | 0.007 | 0.014 |
| 20 | 17th Floor Room 1714 | 0/11 | 0.008 | 0.007 | 0.010 |
| 21 | Sill Area and Files | Not In Scope |  |  |  |
| 22 | 16th Floor Room 1605 | 0/9 | 0.005 | N/U | 0.011 |
| 23 | Ceiling Area | Not In Scope |  |  |  |
| 24 | Ceiling Area | Not In Scope |  |  |  |
| 25 | 16th Floor Mail Room (N wall) | 0/9 | 0.005 | N/U | 0.011 |
| 26 | 16th Floor South Hall | 0/11 | 0.010 | N/U | 0.016 |
| 27 | 16th Floor South Hall | 0/15 | 0.010 | N/U | 0.016 |
| 28 | Ceiling Area | Not In Scope |  |  |  |
| Wall Moisture and Airborne Particulate Sample Results |  |  |  |  |  |
| Location ID# | Location/ Area | % Water WME (surface/ subsurface) | Particulate Concentrations Pre-Removal (mg/m³) |  | Particulate Concentrations During Removal (mg/m³) |
|  |  |  | TSI DustTrak | MIE PDR | MIE PDR |
| 29 | 15th Floor Kirby Office | 0/11 | 0.009 | N/U | N/U |
| 30 | 15th Floor Room 1503 | 0/11 | 0.006 | N/U | N/U |
| 31 | 15th Floor (C Kolpanowski) | 0/9 | 0.007 | N/U | N/U |
| 32 | 15th Floor Axiotis Office | 0/11 | 0.009 | N/U | N/U |
| 33 | 14th Floor (D Jones) | 0/11 | 0.005 | N/U | 0.008 |
| 34 | 14th Floor (J Szilagy) | 0/11 | 0.004 | N/U | 0.005 |
| 35 | 14th Floor Unoccupied Space | 0/9 | 0.011 | N/U | N/U |
| 36 | 14th Floor (D Berry) | 0/11 | 0.006 | N/U | N/U |
| 37 | 12th Floor Room 1208 | 0/9 | 0.006 | N/U | 0.003 |
| 38 | 12th Floor Room 1211 | 0/9 | 0.008 | N/U | 0.110 |
| 39 | Ceiling Area | Not In Scope |  |  |  |
| 40 | 11th Floor Room 1107 | 0/11 | 0.006 | N/U | 0.018 |
| 41 | 11th Floor Room 1108 | 0/9 | 0.004 | N/U | 0.032 |

| Wall Moisture and Airborne Particulate Sample Results |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Location ID# | Location/ Area | % Water WME (surface/ subsurface) | Particulate Concentrations Pre-Removal (mg/m³) |  | Particulate Concentrations During Removal (mg/m³) |
|  |  |  | TSI DustTrak | MIE PDR | MIE PDR |
| 42 | 11th Floor (Leary/Lee) | 0/9 | 0.008 | N/U | 0.007 |
| 43 | 11th Floor Room 1124A | 0/9 | 0.007 | N/U | 0.138 |
| 44 | 10th Floor Room 1006 | 0/9 | 0.008 | N/U | 0.034 |
| 45 | 9th Floor Room 907 | 0/9 | 0.008 | N/U | 0.088 |
| 46 | 9th Floor (Carboy) | 0/11 | 0.004 | N/U | 0.034 |
| 47 | 7th Floor Auditor Area | 0/9 | 0.004 | N/U | 0.020 |
| 48 | 7th Floor Room 707 | 0/9 | 0.005 | N/U | 0.029 |
| 49 | Ceiling Area | Not In Scope |  |  |  |