ITEM# 1111404A   MICROWAVE VEHICLE DETECTOR

Description:

Furnish and install a microwave vehicle detector at the location shown on the plan.

Materials:

- Self tuning.

- 115 (95 to 135) VAC input power.

- Power supply or step down transformer, if other than 115 VAC.

- Light Emitting Diode (LED) visual call indicator.

- Fail safe. A malfunction or damaged detector results in a constant call.

- Operation from -35°F to 160°F (-37°C to 73°C).

- Dimensions: 5”± H, 5”± W, 9”± D (90mm± H, 115mm± W, 190mm± D).

- Weight:  3 lbs. to 5 lbs. (1.3 Kg to 2.2 Kg).

- Water resistant housing.

- Form C relay output.

- Stainless steel mounting hardware.

- Detection:

-Continuous with motion.

-Adjustable range. Up to 200 ft. (60 meters).

-Conical beam. 50 ft. (15 meters) wide at 200 ft. (60 meters).

Construction Methods:

Mount the detector on a fixed structure as recommended by the manufacturer; adjust to provide detection of approaching vehicles only, as shown on plans.

Field drill the structure and install grommet for the cable.

Connect the detector to the power supply in the controller cabinet with the cable.

Insure that cable installed within the fixed structure is protected against weather and damage to cable insulation.

Install LED visual call indicator in the traffic signal cabinet near the microwave hook-up terminals.

Method of  Measurement:

This work will be measured by the number of Microwave Vehicle Detectors furnished, installed, and operating as shown on the plans and accepted by the engineer

Basis of  Payment:

This work will be paid for at the contract unit price each for "Microwave Vehicle Detector". The price will include all materials, parts, power supply, LED visual call indicator and components, tools, labor, and work incidental thereto.