ITEM# 1111470A   TIME CLOCK

DESCRIPTION:

This item shall consist of furnishing and installing a Time Clock/Time Base Coordination (TC/TBC) unit within a traffic signal cabinet in conformity with these specifications.

MATERIALS:

The four circuit TC/TBC shall conform to the current requirements of the Department of Transportation Functional Specifications for Traffic Control Equipment, Four Circuit Solid State Time Clock With Time Base Coordination Option TC/TBC.

CONSTRUCTION METHODS:

The TC/TBC harness shall be connected into the controller cabinet wiring in a neat and orderly manner. Refer to the TC/TBC INSTALLATION REQUIREMENTS included in the special provisions to Item #1108115A - Full Actuated Controller 8 Phase. All connections shall be to terminals. Splices will not be allowed. The TC/TBC hookup complete with pin number, function and cabinet terminal number shall be recorded in the form of a hookup chart, on 216 mm x 280 mm (8.5”x11”) paper. The location, location number, date of revision, project number, TC/TBC manufacturer and model number shall also be recorded on the hookup chart. A program card shall be completed indicating all input steps and settings. Four copies of the hookup chart and program card shall be furnished to the Engineer with one each left in the cabinet. Revise controller timings as shown on the plan.

METHOD OF MEASUREMENT:

This work will be measured for payment by the number of Time Clock/Time Base Coordinators installed; operating in a time base system as designed, and accepted.

BASIS OF PAYMENT:

This work shall be paid for at the contract price each for “Time Clock” which price shall include all materials, labor, tools, 4 copies each of the hookup chart and program card, controller timing changes, and all work incidental thereto.

S:\traffic\1406\signal specs\specs\111470A-time clock.doc