ITEM #1108722A - VEHICLE EMITTER

ITEM #1108724A - PHASE SELECTOR

ITEM #1112410A - DETECTOR (TYPE A)

ITEM #1112470A - PRE-EMPTION SYSTEM CHASSIS

ITEM #1113550A - DETECTOR CABLE (OPTICAL)

SYSTEM DESCRIPTION:

The emergency vehicle traffic signal priority control system shall enable designated vehicles to remotely cause the traffic signal controller to advance to and/or hold a desired traffic signal display by using existing controller functions. The control shall be activated at a minimum distance of 1,800 feet (548.6m) along an unobstructed "line of sight" path. The control shall not terminate until the vehicle is within 40 feet (12.2m) of the detector or at the intersection.

The system shall consist of the following components:

- Vehicle Emitter which shall be mounted on the emergency vehicle and shall transmit optical energy signals only in the forward direction. If the municipality presently uses optical pre-emption, the emitters shall be of the same manufacture currently used by the Town.

- Phase Selector (minimum 2 channel) which shall cause the signal controller to advance to and/or hold the desired traffic signal display for the emergency vehicle. A pre-emption system chassis shall house two phase selectors.

- Optical Detector which shall be mounted on or near a traffic signal and shall receive the optical energy signals generated by the Vehicle Emitter.

Detector (Type A), 1 Direction, 1 Channel

- Detector Cable (Optical).

System Operation:

- The operating sequence shall be initiated when the optical detector receives the required optical energy signal from the Emitter.

- The phase selector shall cause the traffic signal controller to advance to and/or hold the desired traffic signal display for the emergency vehicle.

- The phase selector shall cause the controller to advance to and/or hold the desired traffic signal display even if the optical energy signals cease before the desired display is obtained.

- The phase selector shall allow the traffic signal controller to resume normal operation within ten seconds after optical energy signals cease if the optical energy signals cease after the desired traffic signal display is obtained.

- The phase selector shall not respond to optical energy signals from an emergency vehicle if it is already processing optical energy signals from another emergency vehicle.

System Components:

A.	Vehicle Emitter:

The emitter assembly consists of an emitter and power supply and an emitter control switch assembly. The emitter assembly is mounted on a vehicle and produces a flashing optical signal when in operation.

- Shall operate on ten to fifteen volts DC input voltage, but shall not be damaged by input voltage surges up to twenty-five volts DC.

- Shall be controlled by a single on/off switch that requires no other adjustments by the operator. The on/off condition shall be indicated by a light located adjacent to the switch.

- Shall be automatically disabled or de-activated by one or a combination of the following: seat switch, emergency brake switch, door switch, transmission safety switch.

- Shall operate over an ambient temperature range of minus 30O F. to plus 165O F.

(minus 34O C to plus 74O C)

- Shall operate in 5 to 95 % humidity.

- Shall be a pulsed optical energy source with a controlled repetition rate of 10Hz to 14 Hz and adjustable intensity.

- Shall not generate voltage transients on the battery input line which exceed battery voltage by more than four volts.

- Shall produce optical energy in a cone of not more than 90 degrees horizontal and not more than 30 degrees vertical. The detectors and/or phase selector shall not sense a pre-emption signal from an emitter outside this cone.

B.	Optical Detector:

The optical detector receives the high intensity optical pulses produced by the emitter. These optical energy pulses are transformed by the detector into appropriate electrical signals which are transmitted to the phase selector. The optical detector is mounted at or near the intersection in a location which permits an unobstructed line of sight to vehicular approaches. The units may be mounted on signal span wires, mast arms or other appropriate structures.

- Shall be of solid state construction.

- Shall operate over an ambient temperature range of minus 30O F. to plus 165O F.

(minus 34O C to plus 74O C)

- Shall have internal circuitry potted in a semi-flexible compound to ensure moisture resistance.

- Shall operate in 5 to 95 % humidity.

- Shall have a cone of detection of not more than 13 degrees. The detector and/or phase selector shall not sense a pre-emption signal from an emitter outside this cone.

C.	Phase Selector:

The phase selector supplies power to and receives electrical signals from the optical detector. When detector signals are recognized as a valid call, the phase selector causes the signal controller to advance to and/or hold the desired traffic signal display. This is accomplished by activating the pre-empt input to the controller.

The phase selector is capable of assigning priority traffic movement to one of two channels on a first-come, first-serve basis. Each channel is connected to select a particular traffic movement from those normally available within the controller. Once a call is recognized, "commit to green" circuitry in the phase selector functions so that the desired green indication will be obtained even if optical communication is lost. After serving a priority traffic demand, the phase selector will release the controller to follow normal sequence operation.

- Shall include an internal power supply to supply power to the optical detectors.

- Shall have minimum two-channel operation with the capability of interfacing with an additional phase selector for expansion of channels of operation.

- Shall have adjustable detector range controls for each channel of operation, from 40 feet (12m) to 1800 feet (548m).

- Shall have solid state indicator lights for power on and channel called.

- Shall operate over an ambient temperature range of minus 30O F. to plus 165O F.

(minus 34O C to plus 74O C)

- Shall operate in 5 to 95 % humidity.

D.       Pre -Emption System Chassis:

- Card cage/slot shall provide all the necessary hardware and harnessing required to allow simple wiring of phase selector to detector outputs and controller inputs..

- Shall have harness to carry 115VAC and card outputs.

- Shall include terminal block/strip for connecting the detectors.

E.	Detector Cable (Optical):

1.	3-Conductor cable with shield and ground wire.

2.	AWG #20 (7x28) stranded.

3.	Individually tinned copper strands.

4.	Conductor insulation: 600 volt, 167O F. (75 deg. C).

5.	1 Conductor-yellow; 1 Conductor-blue; 1 Conductor-orange.

6.	Aluminized mylar shield tape or equivalent.

7.	AWG #20 (7x28) stranded uninsulated drain wire

8.	DC resistance not to exceed 11.0 ohms per 1000 feet (305m).

9.	Capacitance from one conductor to other two conductors and shield not to exceed        48 pf/ft. (157pf/m).

10.	Jacket: 600 volts, 176O F. (80 deg. C), minimum average wall thickness – 0.045” (1.14mm).

11.	Finished O.D.:  0.3” (7.62mm) max.

System Interface:

System shall be capable of operating in a computerized traffic management system when appropriate interfacing is provided by the computer supplier.

General:

The Contractor shall furnish the manufacturer the phasing diagrams indicating controller sequence and timing.

The Contractor shall secure from the manufacturer a guarantee for the equipment for a period of sixty (60) months, which time shall commence from the date of delivery. Manufacturer shall certify upon request that all materials furnished will conform to this specification. The manufacturer or his designated representative shall be responsible for determining and setting all required range and emitter intensity for the emergency vehicle operation.

Construction Methods:

All equipment except the vehicle emitter assembly shall be installed and wired in a neat and orderly manner in conformance with the manufacturers’ instructions. The vehicle emitter assembly shall be delivered to a designated town representative. Installation of the vehicle emitter assembly shall be the responsibility of the town.

Traffic signals owned and maintained by the State that have optical pre-emption equipment owned and maintained by the town shall have an Auxiliary Equipment Cabinet (AEC) attached to the controller cabinet. The optical pre-emption equipment shall be housed in the AEC.  Traffic signals owned and maintained by the town do not require an AEC to house the pre-emption equipment.

Detector cables shall be continuous with no splices between the optical detector and the AEC.

Detector locations shown on the plan are for illustration purposes only. Exact location shall be determined by the contractor or the designated representative for the best possible line of sight.

If not present in an existing traffic controller cabinet, the following  items shall be installed and connected, in conformance with the current Functional Specifications for Traffic Control Equipment, “D” Cabinet Requirements (Pre-emption Type):

- Controller “D” harness and adapter.

- Pre-emption termination panel with terminal block and relay bases.

- Pre-emption disconnect switch, mounted on the emergency switch panel (on inside of cabinet door).

- Pre-emption test buttons, mounted on the pre-emption termination panel.

All connections from the phase selector to the “D” harness and to the cabinet wiring shall be made at the termination panel. The termination panel shall have AC+ Lights, AC-, and a switched logic ground. The switched logic ground feeds all the pre-empt inputs to the phase selector. When switched off by the pre-emption disconnect switch, the traffic controller shall not be affected by pre-empt calls from the optical pre-emption system. A minimum of two test buttons shall be provided. If there are more than two pre-empt runs, a button for each shall be installed. A chart or print out indicating the program steps and settings shall be provided along with the revised cabinet wiring diagrams.

Test the Pre-emption System at the semi-final inspection According to the following Guidelines:

- Notify the system owner/user, such as the municipal fire chief or public works director, of the scheduled inspection

- Request a fire department representative and an emergency vehicle, which has an emitter to conduct the test. If not available, the contractor shall provide an emitter.

- In the presence of the Engineer and the municipal representative, test each pre-empted approach with the emergency vehicle. Test the following items of the system:

- Confirm that the emitter activates the phase selector and the phase selector activates the correct pre-emption input to the controller.

- Confirm adequate range. The traffic signal must be pre-empted to green sufficiently in advance of the emergency vehicle arrival. The vehicle emitter shall initiate pre-emption at a minimum distance of 1800 FT. (548.6m).

- Confirm there are no false calls. Keep the emitter active as the emergency vehicle passes through the intersection. No other optical detectors shall sense the strobe.

- Document the test. Provide the Engineer and, upon request, the municipality copies of the test results.

If a malfunction is found or the system needs adjustment (such as range, emitter intensity, or detector location), schedule a follow-up test. Repeat the above steps for all approaches that did not pass.

All adjustments such as emitter intensity, phase selector range, sensitivity, detector placement, shall be made at the intersection by the contractor so that the optical pre-emption operates correctly with other major manufacturers' equipment currently owned by the town.

Method of Measurement:

Optical Detectors, Phase Selectors, System Chassis will be measured for payment by the number of each supplied, installed and accepted. Detector Cable (Optical) will be measured by the number of linear feet (meters) supplied, installed and accepted. Vehicle Emitters will be measured by the number of each supplied to the Town and accepted.

Basis of Payment:

Payment for Optical Detectors, Phase Selector, System Chassis and Detector Cable (Optical) will include the item unit cost, including all manufacturer's required mounting hardware and the cost of installation and supervision by the manufacturer or his designated representative, including travel and subsistence, and all materials, equipment and labor incidental thereto. Payment for termination panel, “D” harness, test buttons, program chart (or print out) and revised cabinet wiring diagrams shall be included in the item PRE-EMPTION SYSTEM CHASSIS. Payment for Vehicle Emitters will include the item unit cost only.

Pay Items					Pay Units

Vehicle Emitter				Ea.

Detector (TYPE A)				Ea.

Phase Selector					Ea.

Detector Cable (Optical)			L.F. (m)

Pre-Emption System Chassis			Ea.