ITEM #0000382A – METRO-NORTH ACCOMMODATION BUILDING

[Metro-North Accommodation Building is based on Metro-North requirements for a temporary ticket office.  Confirm that this is adequate for each Project.]

Description:

Under this item, the Contractor shall provide temporary facilities for the exclusive use of Metro-North and its representatives. The Contractor shall (1) set-up all temporary facilities on the Project Site; (2) connect all temporary facilities to all utilities and support services; and (3) make all temporary facilities fully functional and acceptable to the Engineer prior to the start of demolition of the existing building.

Temporary facilities shall remain on the Project Site for the duration of the Project, including Contract time extensions if any occur, and if required for a maximum of 30 days thereafter.

The State will provide the following materials available to the Contractor on the Project Site to establish the temporary facilities:

1.	[List all applicable materials.]

The State will make the following materials available to the Contractor for pickup at the ConnDOT Rocky Hill Stores Facility to establish the temporary facilities:

1.	[List all applicable materials.]

The State will relocate personnel and materials such as office furniture and equipment from the existing facility into the temporary trailers.  Upon completion of the Project, the State will return said items back to the renovated facility.

The Engineer will approve the materials to be installed by the Contractor, unless otherwise noted.

Materials:

Temporary Power: The Contractor shall include $500.00 in its Contract bid price for electrical utility costs associated with the installation of the temporary power service to the temporary facilities as shown on the plans.  If the Contractor provides written billing documentation at a later date that indicates that the utility costs exceed the estimated amount, the Engineer will adjust the Contractor’s bid to reflect the difference by construction order.  The Contractor will be compensated only for the difference between the billed amount and the estimated amount.  The Engineer will not allow any additional Contractor markup.

The Contractor is responsible for providing temporary power service to its own trailer at its own expense.

Temporary Ticket Office Trailer: A Metro-North Ticket Office, a Bathroom, and a Waiting Room with a Coffee Concession Area will operate from the Temporary Ticket Office Trailer.  The Contractor shall submit a floor plan for the trailer to the Engineer and to Metro-North for approval prior to fabrication.  The trailer will conform to all applicable codes identified in Form 816 Article 1.20-1.02.13 and the following specific requirements.

The trailer shall be a minimum of 60-feet long by 12-feet wide with a minimum ceiling height of 7 feet.  Aluminum siding shall be installed on the exterior of the trailer and vinyl floor tile shall be installed on the floor of the trailer.

Windows shall open and close conveniently, shall be sufficient in number and sized to provide adequate light and ventilation, and shall be fitted with locking devices, blinds, and screens.

The trailer shall have 2 operable entrances with awnings overhead.  Stairs, ramps, handrails, and landings shall be provided to meet all applicable building codes.  The entrance doors shall be fitted with locks and keyed-alike deadbolt locks.  Locksets shall be Model No. [x] as manufactured by [x].  No “Or Equals” will be permitted.

Overhead fluorescent lights shall be provided.  Each area of the trailer shall have its own overhead electrical light or lights, as appropriate.  Sufficient exterior wall-mounted light fixtures shall be installed to illuminate the stairs and ramps from each exterior door.

The Contractor shall install a heating, air conditioning and ventilation system capable of maintaining a temperature between 68 and 80 degrees F, if not integral to the trailer.

The trailer shall be pre-wired to an electrical distribution panel to serve the electrical requirements of the trailer such as lighting, general outlets, and computer outlets, and meet the following specifications: 100A, 120/240 Volt, single phase.  All panels shall include a main circuit breaker and branch circuit breakers of the size and quantity required.  The Contractor shall conform to all applicable requirements of the NEC.

The Contractor shall provide a fire alarm system and portable fire extinguishers as required by the authorities having jurisdiction.  Metro-North will provide a burglar alarm system prior to occupancy.

The Contractor shall provide a 10-foot by 15-foot Ticket Office with the following specific requirements:

(a)	A 24-inch by 36-inch by ½-inch thick Lexan ticket window with talk holes, mounted 42-inches above finish floor.  A laminated window countertop shall be incorporated into the ticket window protruding 6-inches into the Ticket Office and 9-inches into the Waiting Room.

(b)	A window shade on the Ticket Office side of the ticket window.

(c)	A 12-inch deep by 24-inch long laminated shelf shall be installed 6-inches below the ticket window countertop in the Ticket Office.

(d)	A 5-foot by 36-inch by 42-inch high cabinet shall be installed directly to the right of the ticket window in the Ticket Office.  The cabinet shall include a laminated top, lower shelving and storage space.

(e)	A 5-foot long by 30-inch deep built-in desk with a laminated top and drawers and file drawers in the Ticket Office.

(f)	(1) 120V dedicated circuit dual receptacle electrical outlet by the ticket window, (1) 120V dual receptacle electrical outlet by the ticket window, and (2) dual receptacle electrical outlets by the desk.

The Contractor shall provide a 5-foot by 6-foot Bathroom with a portable chemical toilet and sink.  The Contractor shall install and maintain water and sanitary service to serve the water and sanitary needs of the bathroom.

The Contractor shall provide a 10-foot by 25-foot Waiting Room with a small Coffee Concession Area.  The Contractor shall provide (2) 10-foot long wooden benches.  The coffee concession requires a 220V 30 Amp electrical outlet capable of powering a 6600 Watt coffee brewer.  The Contractor shall install and maintain water and sanitary service to serve the water and sanitary needs of a sink that will be provided by the Concession Owner.

The Contractor shall install a trailer skirt between the bottom of the trailer and grade.  The skirt shall be made of Texture 111, or equal.

Construction Methods:

Installation: The Contractor shall locate the temporary facilities where shown on the plans and as directed by the Engineer.  The temporary facilities shall be installed in accordance with Form 816 Article 1.08.02, as supplemented herein.

The Contractor shall level and secure the trailer as directed by the Engineer and in accordance with manufacturer’s recommendations.

The Contractor shall provide applicable utility extensions to the trailer.  Water service and sanitary sewer extensions for the trailer shall be protected from freezing by installation at 48 inches below grade, and with the proper use of insulation and electric heat tape for locations above the frost line.

The Contractor shall maintain, including janitorial service and toilet dumping services a minimum of once per week, the Temporary Ticket Office Trailer installed under the above requirements at all times during occupancy by Metro-North.  The Contractor shall coordinate with Metro-North for maintenance access.  Ownership and liability of the Temporary Ticket Office Trailer shall remain with the Contractor.

Project Closeout: When the Engineer determines that the temporary facilities are no longer needed, the Contractor shall:

- Remove the Temporary Ticket Office Trailer from the Project Site within 14 calendar days of their abandonment;

- Restore the Project Site to its original condition.

Method of Measurement:

This item will be paid for at the contract lump sum price for “Metro-North Accommodation Building” complete.

Basis of Payment:

This item will be paid for at the contract lump sum price for “Metro-North Accommodation Building,” which price shall include all administrative and procedural requirements, material, equipment, labor, and work incidental thereto.

PAY ITEM							PAY UNIT

Metro-North Accommodation Building			LS