ITEM #0952001A – SELECTIVE CLEARING AND THINNING

Section 9.52 is amended as follows:

Article 9.52.03 – Construction Methods is supplemented as follows:

Where directed by the Engineer, materials to be cut, trimmed or removed shall be those items that restrict visibility to an extruded aluminum sign to less than 800 ft.  The entire sign will be visible for 800 ft measured from the center of the right-travel lane approaching the sign, as viewed from a 3.5 ft height above the roadway.

Where directed by the Engineer, materials to be cut, trimmed or removed shall be those items that restrict visibility to a sheet aluminum sign to less than 200 ft.  The entire sign will be visible for 200 ft measured from the center of the right-travel lane approaching the sign, as viewed from a 3.5 ft height above the roadway.

This work shall be completed prior to installing the sign panel onto the vertical supports.

All trees scheduled to be removed shall be visibly marked or flagged by the Contractor at least seven days prior to the cutting of such trees.

The Engineer will inspect the identified trees and verify the limits of clearing and thinning prior to the Contractor proceeding with his cutting operation.