ITEM #1105001A   -   1 WAY, 1 SECTION SPAN WIRE  TRAFFIC SIGNAL

ITEM #1105003A   -   1 WAY, 3 SECTION SPAN WIRE  TRAFFIC SIGNAL

ITEM #1105006A   -   2 WAY, 1 SECTION SPAN WIRE  TRAFFIC SIGNAL

ITEM #1105007A   -   2 WAY, 3 SECTION SPAN WIRE  TRAFFIC SIGNAL

ITEM #1105011A   -   3 WAY, 3 SECTION SPAN WIRE  TRAFFIC SIGNAL

ITEM #1105014A   -   4 WAY, 1 SECTION SPAN WIRE  TRAFFIC SIGNAL

ITEM #1105015A   -   4 WAY, 3 SECTION SPAN WIRE  TRAFFIC SIGNAL

ITEM #1105101A   -   1 WAY, 1 SECTION MAST ARM TRAFFIC SIGNAL

ITEM #1105103A   -   1 WAY, 3 SECTION MAST ARM TRAFFIC SIGNAL

ITEM #1105107A   -   2 WAY, 3 SECTION MAST ARM TRAFFIC SIGNAL

ITEM #1105180A   -   1 WAY, 1 SECTION BI-COLORED ARROW

ITEM #1105203A   -   1 WAY, 3 SECTION POLE MOUNTED TRAFFIC SIGNAL

ITEM #1105303A   -   1 WAY, 3 SECTION PEDESTAL MOUNTED TRAFFIC signal

item #1105490a – 1 way, 1 section traffic signal sign mounted

Article 11.05.03 – Construction Methods:

In the second paragraph, delete the last sentence (“A balance adjuster shall…”).

Add the following paragraphs:

Circular indications that have an identification mark (such as an arrow) on the top of the lens shall be installed with that mark at the 12 o-clock position.

Article 11.05.05 – Basis of Payment:

In the first sentence of the first paragraph, delete “balance adjuster,”.

Article M.16.06 - Traffic Signals

Sub Article 3 - Housing:

In the last sentence, between the words “housing” and “shall” add “and all internal hardware”.

Add the following after the last paragraph.

Each section of the housing shall be provided with a removable visor. The visor shall be the cap type, unless otherwise noted on the plan. The visor shall be a minimum .05 inch (.13 mm) thick. The visor shall be the twist on type and secured to the signal by four equidistant flat tabs screwed to the signal head.

Sub Article 4 - Brackets:

Add the following at the end of the last paragraph:

Backplates shall be 5” wide and louvered.

Install a 2” wide fluorescent yellow retroreflective strip (Type XI sheeting) along the perimeter of the face of the backplate.

Replace the last paragraph with the following:

When indicated on the plans, a backplate of dimensions, as shown on the plans, constructed of 5052-H32 aluminum alloy sheet between 0.050-in to 0.065-in thickness meeting the requirements of ASTM B209 shall be attached to the signal head housing. The front surface of backplate per MUTCD shall have a dull black finish to minimize light reflection and to increase contrast between the signal indication and its background.

Delete Sub Article 5 - Optical Unit and Sub Article 6 – Lamp Socket and replace with the following:

Optical Unit, Light Emitting Diode:

(a) General:

Only Optical Units that meet the requirements contained herein supplied by the below manufacturers that have been tested by the Department’s Signal Lab will be accepted. Final approval for model numbers will be done at the time of the catalog cut submittals.

Duralight					GE Lighting Solutions

Trastar, Inc.					Corporate Headquarters

860 N. Dorothy Dr., Suite 600 		1975 Noble Road Building 338E

Richardson, TX 75081			East Cleveland, OH 44112-6300

Dialight 					Leotek

1501 Foute 34 South				726 South Hillview Drive

Farmingdale, NJ 07727			Milpitas, CA 95035

The materials for Light Emitting Diode (LED), Optical Unit, circular and arrow, shall conform to the following:

- The ITE Performance Specification for Vehicle Traffic Control Signal Heads – Light Emitting Diode (LED) Circular Signal Supplement for circular indications dated June 27, 2005.

- The ITE Performance Specification for Vehicle Traffic Control Signal Heads – Light Emitting Diode (LED) Vehicle Arrow Traffic Signal Supplement for arrow indications dated July 1, 2007.

Section 4, Adjustable Traffic Signals and General Housing sections of the Department of Transportation Functional Specifications for Traffic Control Equipment, current edition governs. Where the Department of Transportation Functional Specifications conflict with this Special Provision or the 2005/2007 ITE Performance Specifications, this Special Provision and the 2005/2007 ITE Performance Specifications shall govern.

The Optical Unit shall have an Incandescent look and be made up of a smooth surfaced outer shell, multiple LED light sources, a filtered power supply and a back cover, assembled into a sealed unit. The Optical Unit shall be certified as meeting the 2005/2007 ITE Specifications by Intertek Testing Services, Inc. (ITSNA, formerly ETL) or another organization currently recognized by the Occupational Safety and Health Administration (OSHA) as a Nationally Recognized Testing Laboratory (NRTL.) The Optical Unit shall perform to the requirements of the ITE Specification for a minimum of 60 months.

A “Swing Test” will be performed by the Department to ensure no significant dimming or blanking occurs, until the lamp is obscured by the visor. All L.E.D Lamps will be subjected to further field testing for reliable operation.

The Arrow Optical Unit shall be “Omni-Directional” so that it may be oriented in a right, left or straight configuration without degradation of performance.

(b) Electrical Requirement:

Operating voltage:

80 to 135 Volts AC with cutoff voltage (no visible indication) below 35Volts AC.

Power requirements:

Circular Indications:	   12”, (300 mm) – no more than 16 Watts

Circular Indications:    8”, (200mm) - no more than 16 Watts

Arrows Indications:    12”, (300mm) - no more than 16 Watts

Power Supply:

Fused and filtered to provide excess current protection and over voltage protection from electrical surges and transient voltages.

(c) Photometric Requirement:

Beam Color:

Meet 2005/2007 ITE Specifications

(d) Mechanical Requirements:

Diameter:

The Circular Optical Unit shall fit into standard 12” (300mm) or 8” (200mm) housing.

The Arrow Optical Unit shall fit 12” (300mm) housings only.

Enclosure:

UV (Ultraviolet) stabilized polycarbonate back cover.

Clear lens cover for all Red, Yellow and Green Circular Optical Units.

For Arrow Optical Units the arrow indication segment of the lens shall be clear.

Enclosure sealed and waterproofed to eliminate dirt contamination and be suitable for

installation in all weather conditions.

Clearly mark on the housing the following information:

- Manufacturer & model number

- Date of manufacture (must be within one year of installation)

The model number shall end with the number of LEDs used to comprise the unit as the last

digits of the model number. Example, if the unit comprised of 3 LEDs and the model is

x12y, then the new model number shall read x12y3.

Operating temperature:

Meet 2005/2007 ITE Specification

Wiring: L.E.D. lamps shall have color coded 16 AWG wires for identification of heads as follows:

| RED L.E.D. Lamps | RED with WHITE neutral |
| --- | --- |
| YELLOW L.E.D. Lamps | YELLOW with WHITE neutral |
| GREEN L.E.D. Lamps | GREEN or Brown with WHITE neutral |
| RED L.E.D. ARROWS | RED/WHITE with WHITE neutral |
| YELLOW L.E.D. ARROWS | YELLOW/WHITE with WHITE neutral |
| GREEN L.E.D. ARROWS | GREEN/WHITE or BROWN/WHITE with WHITE neutral |
| GREEN/YELLOW LE.D ARROWS | GREEN/WHITE or BROWN/WHITE, YELLOW/WHITE, with WHITE neutral |

Wires shall be terminated with a Block Spade, 6-8 stud/ 16-14 wire size.

All Circular Optical Units shall be supplied with a minimum 40” pigtail and all Arrow Optical Units Supplied with a minimum 60” pigtail.

Sub Article 9 - Painting:

Third coat:

Replace with the following:

The housing, housing door, the back surface of the backplate, and all brackets and hardware shall be painted black by the manufacturer.  The color shall be No. 17038, Federal Standard No. 595.

At intersections at Merritt Parkway interchanges, the housing and all brackets and hardware shall be painted dark green by the manufacturer. The color shall be No. 14056, Federal Standard No. 595.

The outside of the visors shall have a dull black finish that meets Federal Specification TT-E-527.

The inside of the visors per MUTCD shall have a dull black finish to minimize light reflection and to increase contrast between the signal indication and its background. The dull black finish shall meet Federal Specification TT-E-527.