ITEM #1113398A - CABLE CLOSURE (TYPE A)

Work under this item shall conform to the requirements of section 11.13 supplemented and amended as follows:

Article M.16.14  - Control Cable:

Subarticle M.16.14 - 5 - Cable Closure:  After the only paragraph add the following:  The cable closure installed on the figure 8 cable and used to terminate interconnect shall be designated as Type A. This closure shall be weather resistant, contain an inner shield bond and grounding continuity system. All external parts shall be stainless steel.  The closure shall be provided with 3 or 4 entrances, dependent on the number of cables terminated in the closure. The main entrance shall accommodate cable with a diameter of 0.4"(10mm) to 1.0"(25mm) and the branch entrance shall accommodate a cable with a diameter of 0.4"(10mm) to 1.0"(25mm). The size of the closure shall be: length 17" (425mm) to 24” (610mm), width 3" (76mm) to 5” (127mm),  height 5" (127mm) to 6.5” (165mm).  Each closure shall be provided with a minimum of 24 terminals, either screw type or pressure type.

Article 11.13.03 - Construction Methods:

Subarticle 11.13.03 - 3 Cable Closures:  After the only paragraph add the following:  The Type A cable closures attached to the figure 8 cable shall be used for to provide an access for the 16/6 pair interconnect to the controller cabinet.  The locations shall be as listed below and as shown on the plans.

Intersection	Location No.	Pole No.

Rt.        	 @ 		CL&P

Rt.        	 @ 		CL&P

Rt.        	 @ 		CL&P

Rt.        	 @ 		CL&P

Rt.        	 @ 		CL&P

Only the conductors designated as "Traffic Signal Interconnect Pairs" in the special provisions to the Communication Cable, shall be cut and terminated in these closures. Connection to traffic signal pairs shall be as shown in the Typical Cable Closure Illustration contained herein. The conductors designated as not used shall not be cut at these closures. The Type A Cable Closures shall be installed approximately 3 feet (0.9 m) from the indicated utility pole, unless otherwise directed by the Engineer.

S:\traffic\1406\signal specs\specs\1113398A-cable closure (type A).doc