# Date:      			        Site:      			Observer:

# Observation of          Co-Instructor:         Role:  I    IA     # of Students:

30 minutes – Demo, Simulated Practice, 2 parts

Start Time:         	        End time:           		Total Time:          Minutes

T:      Objective:  To be able to stop quickly;  Read verbatim (Lead only):

Range setup:  2 lanes on each side of the range; 2 crossed pause-n-go’s in middle of the range;  Correct

T:      Provide Instructions (Lead only)

PART 1 – STOP USING CUE CONES

- On signal, ride down the middle of the range through the crossed pause-n-go’s

- Ride to a start point for stop lane

- On signal and one at a time, approach stopping area at about 15mph in 2nd gear

- Stabilize speed early

- As front wheel passes cue cones, downshift keeping the clutch squeezed, & make a quick stop using both brakes

- Once coached, make the perimeter turn and ride trough the crossed pause-n-go’s

PART 2 – STOP ON INSTRUCTOR SIGNAL

- When an Instructor moves inside stopping area, stop quickly on command

Correct & clearly stated;  Range card diagram used; Student questions delayed until after demo

T:      With class at stop point, Provide Demo Stopping Quickly

CO-INSTRUCTOR:

Start/end in appropriate location;  All safety gear worn;  FINE-C used;  Correct posture;

Correct exercise technique; Correct speed;  Correct shut-down & dismount

LEAD INSTRUCTOR:

Note evaluations and provide signals

    - Keep head and eyes up

    - Use brakes firmly, not grabbing the front brake or locking the rear brake

      - If rear wheel locks, use less pressure next time

    - Shift with precision

    - Maintain a safety margin

Students in position to easily view evaluation points;  Demo & evaluation points synchronized; Intro new signals

T:      Provide Simulated Practice Stopping: Roll off the throttle; Progressive squeeze of front brake lever.

Number of Repetitions      .

Each student demonstrated correct technique;  Corrective feedback provided when necessary

T:      Conduct Exercise (To start, distribute riders evenly)

- Part 1:  use cue cones

T:       Part 2

- Part 2:  use stop signal

| COACHING: | EVALUATE & IDENTIFY | RANGE POSITION / MGT |
| --- | --- | --- |
| By correction Short, clear, positive Prioritized by student needs Two errors max Good communication skills | Used head…feet evaluation sequence Prioritize by student need Individualized feedback How to perform correctly Effective verbal remediation Effective non-verbal remediation Positive reinforcement Hand signals Voice projection, distinct speech | Position to control / evaluate Range in view Visual contact each rider Indicated Positions used Move to next position Position to observe during feedback Position most helpful to students |

| INSTRUCTOR COORDINATION | OTHER EVALUATION POINTS |  |
| --- | --- | --- |
| Staging | ## Conducted correctly (all parts) ## Achieved objective/s ## Achieved other evaluation points Maintain safe learning environment | Range Safety ## Professional appearance ## Positive attitude Appropriate style |

Stage - riders in parking area;  SEE; Under control

T:      Debrief – Focus on exercise objective:  to be able to stop quickly;

Group feedback on evaluation points if needed;   Co-Instructor contributed

T:      End exercise

## Observer Comments:

## Chief Comments: