# Date:      			        Site:      			Observer:

# Observation of          Co-Instructor:         Role:  I    IA     # of Students:

20 minutes – Demo, Simulated Practice

Start Time:         	        End time:           		Total Time:          Minutes

T:      Objective:  To be able to stop quickly in a curve; Read verbatim (Lead only)

Range setup:  2 curved paths marked by lines and cones, with cross traffic in the middle;  Correct

T:      Provide Instructions (Lead only)

- On signal and one at a time, ride into curve in 1st gear or 2nd gear

- After entering the curve, straighten and brake to a stop, going over the line if necessary

- After coaching, select a safe gap and proceed to next line

- Repeat other direction

- On signal, return to parking area

Correct & clearly stated;  Range card diagram used;  Student questions delayed until after demo:

T:      Provide Demo Stopping Quickly in a Curve: (With riders at stop point)

CO-INSTRUCTOR:

Start/end in appropriate location;  All safety gear worn;  FINE-C used;  Correct posture;

Correct exercise technique;  Correct speed;  Correct shut-down & dismount

LEAD INSTRUCTOR:

Note evaluations and provide signals:

- Use outside-inside-outside path

- Straighten before braking

- Keep head and eyes up

- Do not brake too hard

- Have handlebars square when stopped

- Select a safe gap in middle

- Maintain a safety margin

Students in position to easily view evaluation points;  Demo & evaluation points synchronized;  Intro new signals

T:      Provide Simulated Practice - Straighten, THEN Brake:

- Square handlebars;

- Squeeze front brake.

Number of Repetitions:

Each student demonstrated correct technique;  Corrective feedback provided when necessary

T:      Conduct Exercise, with Reversal

- Run in both directions, repeat as necessary

| COACHING: | EVALUATE & IDENTIFY | RANGE POSITION / MGT |
| --- | --- | --- |
| By correction Short, clear, positive Prioritized by student needs Two errors max Good communication skills | Used head…feet evaluation sequence Prioritize by student need Individualized feedback How to perform correctly Effective verbal remediation Effective non-verbal remediation Positive reinforcement Hand signals Voice projection, distinct speech | Position to control / evaluate Range in view Visual contact each rider Indicated Positions used Move to next position Position to observe during feedback Position most helpful to students |

| INSTRUCTOR COORDINATION | OTHER EVALUATION POINTS |  |
| --- | --- | --- |
| Staging | ## Conducted correctly (all parts) ## Achieved objective/s ## Achieved other evaluation points Maintain safe learning environment | Range Safety ## Professional appearance ## Positive attitude Appropriate style |

Stage riders in parking area;  SEE;   Under control.

T:      Debrief FOCUS on objective: To be able to stop quickly in a curve;

Group feedback on evaluation points if needed;   Co-Instructor contributed

T:      End exercise

## Observer Comments:

## Chief Comments: