# Date:      			        Site:      			Observer:

# Observation of          Co-Instructor:         Role:  I    IA     # of Students:

30 minutes – Demo, Simulated Practice, 2 parts

Start Time:         	        End time:           		Total Time:          Minutes

T:      Objective: To be able to avoid hazards by swerving or stopping quickly;  Read verbatim (Lead only)

Range setup: 2 separate barriers and an offset weave on outside perimeter; The cue cones are 15’ from the barrier and 3’ apart; The weave cones are 30’ apart with a 3’ offset  Correct

T:      Provide Instructions (Lead only)

Part 1 – Swerve

- On signal and one at a time, approach barrier at around 12 mph in 2nd gear

- Maintain a steady speed

- Swerve in direction signaled without braking

- When straight, downshift and stop next to RiderCoach

- After coaching, practice the offset weave and return to end of same line.

Part 2 – Swerve or Brake

- After some time, the RiderCoach will add a stop signal, providing 3 options: swerve left, swerve right, brake in straight line

Correct & clearly stated;   Range card diagram used;  Student questions delayed until after demo

T:      Provide Demo Avoiding Hazards: (With riders at coach position)

CO-INSTRUCTOR:

Start/end in appropriate location;  All safety gear worn;  FINE-C used;  Correct posture;

Correct exercise technique;  Correct speed;   Correct shut-down & dismount

LEAD INSTRUCTOR:

Note evaluations and signals for Part 1:

- Keep head and eyes up

- Keep speed under control

- Keep body upright, independent of motorcycle lean

- Maintain steady speed when swerving

- Do not brake while swerving

- In weave, maintain steady speed

Note evaluations and signals for Part 2

- When stopping, downshift and brake smoothly in a straight line

- In weave, maintain steady speed

Students in position to easily view evaluation points;  Demo & evaluation points synchronized;  Intro new signals

T:      Provide Simulated Practice Swerve:

- Press in direction of swerve;

- Press to straighten.

Number of Repetitions

Each student demonstrated correct technique;   Corrective feedback provided when necessary

T:      Conduct Exercise

- Provide early signals, no surprises

- For Part 2, tell riders that first signal will be a stop signal, then signals will be random

| COACHING: | EVALUATE & IDENTIFY | RANGE POSITION / MGT |
| --- | --- | --- |
| By correction Short, clear, positive Prioritized by student needs Two errors max Good communication skills | Used head…feet evaluation sequence Prioritize by student need Individualized feedback How to perform correctly Effective verbal remediation Effective non-verbal remediation Positive reinforcement Hand signals Voice projection, distinct speech | Position to control / evaluate Range in view Visual contact each rider Indicated Positions used Move to next position Position to observe during feedback Position most helpful to students |

| INSTRUCTOR COORDINATION | OTHER EVALUATION POINTS |  |
| --- | --- | --- |
| Staging | ## Conducted correctly (all parts) ## Achieved objective/s ## Achieved other evaluation points Maintain safe learning environment | Range Safety ## Professional appearance ## Positive attitude Appropriate style |

Stage Keep near-side riders in place; Direct far-side riders to turn left after last swerve-stop and join the near group.

SEE;   Under control

T:      Debrief FOCUS on objective:  To be able to avoid hazards by swerving or stopping quickly;

Group feedback on evaluation points if needed;   Co-Instructor contributed

T:      End exercise

## Observer Comments:

## Chief Comments: