# Date:      			        Site:      			Observer:

# Observation of          Co-Instructor:         Role:  I    IA     # of Students:

20 minutes – Demo (optional)

Start Time:         	        End time:           		Total Time:          Minutes

T:      Objective: To be able to perform basic motorcycle operation smoothly and proficiently;  Read verbatim

Range setup: Quick stop lanes on the interior of range and on near side perimeter; U-turns and swerve area on far side of range. (The cue cones for swerve 13’ from barrier and 3’ apart; the 20’ boundary line for U-turn is used.);  Correct

T:      Provide Instructions, explaining path of travel (Lead only)

- On signal and one at a time, practice quick stops in the 2 lanes provided

- Begin stop when front tire crosses cue cones

- After stopping return to end of line

- On signal, add the U-turns and swerve to path of travel

- Wait until U-turn and swerve areas are open before starting into them

- Return to original line

Correct & clearly stated;  Range card diagram used;  Student questions delayed until after demo

T:      Provide Demo (optional)

CO-INSTRUCTOR:

Start/end in appropriate location;  All safety gear worn;  FINE-C used;  Correct posture

Correct exercise technique;  Correct speed;  Correct shut-down & dismount

LEAD INSTRUCTOR:

Note evaluations and provide signals

For STOPS:

- Use brakes firmly

- Keep head and eyes up

- Avoid anticipating stop point

For U-TURNS

- Turn handlebars and counterweight

- Keep head up and look through path of travel

- Work at completing turns within solid line

For SWERVE

- Keep head and eyes up

- Maintain steady speed in approach and swerve

- Separate braking and swerving

- When straight, stop completely before returning to line.

Students in position to easily view evaluation points;  Demo & evaluation points synchronized;  Intro new signals

T:      Conduct Exercise:

- Initially have riders use both stop lanes;

- Coach improper approach speeds and cue cone anticipation;

- When riders are performing quick stops well, add the U-turn and swerve to path of travel.

| COACHING: | EVALUATE & IDENTIFY | RANGE POSITION / MGT |
| --- | --- | --- |
| By correction Short, clear, positive Prioritized by student needs Two errors max Good communication skills | Used head…feet evaluation sequence Prioritize by student need Individualized feedback How to perform correctly Effective verbal remediation Effective non-verbal remediation Positive reinforcement Hand signals Voice projection, distinct speech | Position to control / evaluate Range in view Visual contact each rider Indicated Positions used Move to next position Position to observe during feedback Position most helpful to students |

| INSTRUCTOR COORDINATION | OTHER EVALUATION POINTS |  |
| --- | --- | --- |
| Staging | ## Conducted correctly (all parts) ## Achieved objective/s ## Achieved other evaluation points Maintain safe learning environment | Range Safety ## Professional appearance ## Positive attitude Appropriate style |

Stage riders at the start point for the U-turn;  SEE;  Under control

T:      Debrief FOCUS on objective: To be able to perform basic motorcycle operation smoothly and proficiently;

Group feedback on evaluation points if needed;   Co-Instructor contributed

T:      End exercise

## Observer Comments:

## Chief Comments: